- Gold jewellery is not usually made of the pure metal, as it is too soft and is also expensive. To make jewellery, an alloy of gold is usually used. The metals added are often cheaper than gold, like silver, copper and nickel, and so the alloy is less expensive. However, sometimes a more expensive metal is added, such as palladium or platinum, to produce different properties. The properties that can be altered by making alloys include hardness, strength, malleability, resistance to corrosion and colour. The table shows the composition of some common alloys of gold.

| Gold alloy | pure gold | green gold | pink gold | red gold | dental gold |
| --- | --- | --- | --- | --- | --- |
| Composition | 100% gold | 75% gold 15% silver 10% palladium | 37.5% gold 20% silver 42.5% copper | 58.5% gold 7.5% silver 34% copper | 75% gold 18% copper 5% aluminium 2% gallium |
| Melting point (C) | 1064 | 990 to 1005 | 1070 to 1075 | 1055 to 1067 | 1025 to 1035 |

The purity of gold objects can be measured in ‘carats’. One carat is 1 part gold in 24 parts of alloy. This means that pure gold is 24 carats and we can calculate the % gold as shown below.

For example: 12 carats = 12/24 = 0.5 = 50% gold

The newer ‘fineness’ system measures purity in parts per thousand. In this system, pure gold is 1000 and an object made of 50% gold is 500. Note that in the ‘fineness’ system, pure gold is often labelled 999 instead of 1000.

- Give two possible reasons for using an alloy of gold rather than pure gold.

- Why might you mix gold with a more expensive metal rather than a less expensive one?

- Dentists sometimes use gold for filling teeth. What special properties would dental gold need to have?

- The gold alloys in the table vary in purity. Which of the metals in the table could be described as:

| i 9 carat | ii 14 carat | iii 18 carat | iv 24 carat |
| --- | --- | --- | --- |

- Calculate the purity of each of the gold alloys in the table using the ‘fineness’ system.

- Explain why the melting point for pure gold is a single figure while all the other melting points are quoted as a range of temperatures.

- The table below shows the temperature changes that occurred as solid lead metal was heated steadily for 10 minutes. During the heating the lead melted.

| Time (min) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature (C) | 20 | 81 | 143 | 205 | 267 | 328 | 328 | 328 | 328 | 368 | 409 |

- What is the melting point of lead?

- Explain how you worked out the melting point from the data in the table.

- What happens to the particles of a liquid as it is heated if no change of state occurs.

I can...

- describe and use some different ways of stating purity

- get relevant information from text.